General Rules (Applies to All Components)

- The structural model is created entirely in Rhino, not in Grasshopper.

- Grasshopper only reads and converts Rhino data into OpenSees format.

- Geometry types must be:

  - Polylines → beams, columns, springs

  - Points → supports, nodal loads

  - 3 or 4 corner surfaces → slabs, walls

- Layer names define element types. Wrong layer = element not recognized.

- Structural properties are assigned using User Text in Rhino.

- If User Text is missing, default values are automatically used.

- Beam and column intersections are automatically split into elements.

- Always press F5 / Recompute after editing Rhino data.

ReadBeam2 (Lines → Beams, Columns, Springs)

- Reads beam, column, and spring elements from polyline geometry.

- Automatically detects intersections and splits members.

- Reads material, section, angle, reinforcement, buckling length, loads, and joint type from User Text.

- joint only controls rotational stiffness (rigid/pin/elastic), not real RC joint behavior.

- Buckling length defaults to original member length unless specified.

- Springs are modeled as elastic elements with axial, shear, and rotational stiffness.

- Grid/axis labels (wickX, wickY, wickZ) are stored for later filtering.

ReadPoint & ReadShell (Points and Surfaces)

ReadPoint

- Reads boundary conditions (supports) and nodal loads from point geometry.

- Requires node coordinates from beam output to match supports correctly.

- Has no preview; correctness is checked after recompute.

ReadShell

- Reads surface elements in two modes:

  - Shell mode → RC slabs, plates (uses thickness, material, surface load)

  - Timber mode → Japanese wall/floor ratio method (uses K and rad, material ignored)

- Surfaces must be 3 or 4 corner surfaces.

- Surface loads can be defined for gravity and seismic load generation.

- Grid labels (wickX, wickY) are stored for filtering.

MakeIndex2 (Filtering Elements)

- Generates element indices belonging to specified layers.

- Can filter by layer and grid/axis labels (wick).

- Beam layers and spring layers cannot be mixed.

- Used for selective visualization and section design.

- Useful for isolating specific members (e.g., steel beams, foundation beams, walls).